Jewelry Design Language (JDL)

Version 0.1  —  Engagement Rings

A formal grammar for parsing, composing, and manufacturing Indian retail jewelry

# 1. The Thesis

Jewelers do not design from nothing. They recombine a finite vocabulary of known elements — shanks, heads, prongs, accents, finishes. A master in Karol Bagh, shown a Pinterest screenshot, silently parses it into this vocabulary and reproduces it. The vocabulary is transmitted through apprenticeship, not written down.

JDL writes it down.

Once the grammar exists, three things become possible that are impossible today: (1) Vision models can parse photos into JDL (a classification problem they solve well) instead of generating 3D geometry (a problem they solve badly). (2) A synthesizer composes the JDL into CAD that is manufacturable by construction — because every primitive is pre-engineered to cast. (3) The JDL itself becomes a portable format jewelers, customers, casting houses, appraisers, and insurers can all read.

## 1.1 What this document is

A first-draft specification of JDL for engagement rings. It covers ~80% of Indian retail engagement ring designs. It is intentionally incomplete — the point is to take this to a master jewelry CAD designer in Jaipur or Mumbai and have them tear it apart, fill gaps, and correct errors. That feedback loop is the first month's work.

## 1.2 What this document is not

Not a final standard. Not a product spec. Not code. It is a linguistic blueprint: the parts of speech, the rules of composition, and worked examples.

# 2. Grammar Overview

JDL is hierarchical. A piece is composed of components. Components have required parameters and optional modifiers. Invalid combinations are not expressible — the grammar rejects them at parse time, which is why generated CAD is guaranteed manufacturable.

## 2.1 Top-level structure

piece: ring
  shank: <shank_element>
  head: <head_element>
  center_stone: <stone_element>       # required for engagement rings
  accent: <accent_element>            # optional, repeatable
  gallery: <gallery_element>          # optional
  finish: <finish_element>
  metal: <metal_spec>
  size: <size_spec>

## 2.2 Design principles

- Every parameter has units. No implicit assumptions.

- Every primitive is parametric — size adjustability, stone substitution, metal change are all variable changes, not redesigns.

- Every primitive has a manufacturability envelope: minimum wall thickness, minimum feature size, maximum overhang. The synthesizer enforces these.

- Every primitive has a material-cost function. The synthesizer outputs estimated gold weight along with the CAD.

# 3. The Vocabulary (v0.1)

Each table below is a category. Each row is a primitive. Each primitive is one hand-built parametric CAD file in the synthesizer library.

## 3.1 Shank Styles

The band of the ring — the part that circles the finger. The most-requested modification category.

| JDL token | Description | Key parameters |
| --- | --- | --- |
| plain_round | Classic round cross-section | width_mm, thickness_mm |
| plain_flat | Flat outer face, rounded inside (comfort fit) | width_mm, thickness_mm |
| plain_square | Square cross-section | width_mm, thickness_mm |
| knife_edge | Tapered to a ridge on top | width_mm, ridge_height_mm |
| cathedral | Shank rises toward the head, supporting it | width_mm, rise_mm, rise_length_mm |
| tapered | Narrows from shoulders toward base | shoulder_width_mm, base_width_mm |
| split_shank_v | Splits into two arms at the head (V pattern) | split_length_mm, split_angle_deg |
| split_shank_parallel | Splits into parallel arms | split_length_mm, arm_width_mm, gap_mm |
| twist | Two or more bands twisted | band_count, twist_angle_deg, band_width_mm |
| infinity | Crossover pattern forming infinity symbol | crossover_point, arm_width_mm |
| bypass | Two arms overlap without joining | overlap_length_mm, gap_mm |
| eternity | Stones set continuously around entire shank | requires accent: pave/channel |
| half_eternity | Stones set around top half only | coverage_deg (typ 180) |
| tapered_baguette | Baguette stones flanking the head | baguette_count, baguette_size |
| milgrain_edge | Plain band with beaded edge detail | width_mm, milgrain_size (fine/medium/bold) |

## 3.2 Head Styles

The part that holds the center stone. The defining visual of an engagement ring.

| JDL token | Description | Key parameters |
| --- | --- | --- |
| solitaire_4prong | Single center stone, 4 prongs | prong_style, prong_length_mm |
| solitaire_6prong | Single center stone, 6 prongs | prong_style, prong_length_mm |
| solitaire_3stone | Center stone flanked by 2 side stones | side_stone_ratio, side_stone_cut |
| halo_single | Center stone surrounded by one ring of accents | halo_stone_count, halo_stone_size_mm |
| halo_double | Two concentric rings of accents | inner_count, outer_count, outer_stone_size_mm |
| halo_floral | Halo shaped like flower petals | petal_count |
| bezel_full | Stone fully enclosed in metal rim | bezel_height_mm, bezel_width_mm |
| bezel_half | Bezel on 2 sides, open on other 2 | bezel_height_mm |
| tension | Stone held by shank pressure (appears suspended) | tension_gap_mm |
| cluster | Multiple stones grouped to appear as one | stone_count, cluster_shape |
| cathedral_trellis | Prongs extend up from shank like arches | arch_height_mm |
| illusion_plate | Stone set in reflective plate to appear larger | plate_shape, plate_size_mm |

## 3.3 Prong Styles

The metal claws that grip the stone. Independent of prong count.

| JDL token | Description | Manufacturing note |
| --- | --- | --- |
| claw_round | Rounded tip, most common | Easiest to cast |
| claw_pointed | Pointed tip, classic look | Risk of snagging fabric |
| claw_flat | Flat top, modern | More metal, higher cost |
| claw_v | V-shaped, for princess/square cuts | Required for corner protection |
| double_claw | Two prongs in place of each one (8-prong look) | Requires closer spacing rules |
| shared_prong | One prong holds two adjacent stones | Used in accent settings only |
| tiger_claw | Textured/notched for grip | Decorative, traditional Indian style |

## 3.4 Center Stone Specification

The stone itself is not a CAD primitive — it is a specification the setting must conform to.

center_stone:
  cut: round_brilliant | princess | cushion | oval | emerald |
       pear | marquise | radiant | asscher | heart
  dimensions:
    diameter_mm: <for round>
    length_mm, width_mm, depth_mm: <for fancy cuts>
  carat_weight: <auto-calculated from dimensions + cut>
  setting_depth_mm: <how much stone sits below shank top>

This is critical: the synthesizer uses these dimensions to generate the stone seat geometry inside the head primitive. The seat is parametric — changing the stone changes the seat automatically.

## 3.5 Accent Patterns

Additional stones set into the shank or around the head. Repeatable — a ring can have multiple accent elements.

| JDL token | Description | Key parameters |
| --- | --- | --- |
| pave | Tiny stones set close with small beads of metal | stone_size_mm, row_count, length_mm |
| micro_pave | Pavé with stones under 1mm | stone_size_mm, length_mm |
| channel | Stones held between two metal walls | stone_count, stone_size_mm, length_mm |
| bar | Stones separated by thin metal bars | stone_count, bar_width_mm |
| bead_set | Stones held by raised beads of metal | stone_count, pattern (linear/cluster) |
| gypsy | Stone set flush into metal (no prongs) | stone_size_mm, count |
| baguette_flanking | Baguette stones on either side of head | baguette_length_mm, count_per_side |
| trillion_flanking | Trillion (triangle) stones flanking head | trillion_size_mm |

## 3.6 Gallery (Under-the-Head Detail)

The visible part of the head when viewed from the side. Often overlooked but a major Indian-market differentiator.

| JDL token | Description |
| --- | --- |
| plain | Solid metal, no openwork |
| open_basket | Simple openwork allowing light through stone |
| filigree | Intricate openwork pattern |
| surprise_stone | Hidden stone set under the center stone |
| heart_cutout | Heart-shaped cutout — popular for proposals |
| infinity_cutout | Infinity symbol cutout |
| floral | Floral openwork, common in South Indian designs |
| cathedral_arches | Arches visible from the side (pairs with cathedral shank) |

## 3.7 Finish

| JDL token | Description |
| --- | --- |
| high_polish | Mirror finish, most common |
| matte | Brushed, non-reflective |
| satin | Between polish and matte |
| hammered | Hand-hammered texture |
| sandblast | Uniform rough texture |
| florentine | Crosshatched engraved lines |
| milgrain_all | Beaded detail on all edges |
| antique_patina | Darkened in recesses to highlight detail |

## 3.8 Metal Specification

metal:
  material: 18k_yellow_gold | 18k_white_gold | 18k_rose_gold |
            22k_yellow_gold | 14k_yellow_gold | platinum_950 |
            silver_925
  plating: none | rhodium | gold                # optional post-finish
  tone_variation: single_tone | two_tone | three_tone
    # for two/three tone, specify which components use which metal:
    # shank: 18k_yellow_gold, head: 18k_white_gold

## 3.9 Size Specification

size:
  standard: IN_12 | IN_14 | IN_16 | ...          # Indian ring sizes
  # OR
  inner_diameter_mm: <exact measurement>
  # OR
  us_size: 6 | 6.5 | 7 | ...

      
resizable: true | false                          # is the design resizable without remake?
resize_range: [min_size, max_size]               # if resizable

# 4. Composition Rules

Not every combination is valid. These rules are enforced by the synthesizer.

### 4.1 Compatibility constraints

- tension head requires shank thickness ≥ 2.5mm (structural integrity)

- eternity shank cannot combine with split_shank_* (geometric impossibility)

- claw_v prongs required for princess, asscher, and emerald cuts (corner protection)

- micro_pave requires metal purity ≤ 18k (22k is too soft to hold bead settings)

- bezel_full head hides stone culet — reduces light return, flag to jeweler

- cathedral shank pairs only with cathedral_trellis or solitaire_* heads

### 4.2 Manufacturability envelope

- Minimum shank thickness: 1.2mm (below this, will bend)

- Minimum prong tip thickness: 0.4mm

- Minimum wall thickness anywhere: 0.8mm (below this, casting fails)

- Minimum gap between accent stones: 0.2mm (stone-to-stone, not counting metal)

- Maximum overhang without support: 1.5mm (beyond this, wax pattern sags)

### 4.3 Material-cost enforcement

Every primitive has a volume function V(params) → mm³. Total metal volume is the sum across primitives minus stone displacement. Gold weight = volume × density. Jeweler receives cost quote before CAD is finalized.

# 5. Worked Examples

Three common Indian retail engagement rings, expressed as JDL.

## 5.1 The classic solitaire

Tiffany-style 6-prong solitaire, 1ct round diamond, plain band. The single most-requested design globally.

piece: ring
  shank:
    type: plain_round
    width_mm: 2.0
    thickness_mm: 1.8
  head:
    type: solitaire_6prong
    prong_style: claw_round
    prong_length_mm: 4.5
  center_stone:
    cut: round_brilliant
    diameter_mm: 6.5
    carat_weight: 1.00
  gallery:
    type: open_basket
  finish:
    type: high_polish
  metal:
    material: 18k_white_gold
    tone_variation: single_tone
  size:
    standard: IN_14
    resizable: true
    resize_range: [IN_10, IN_18]

## 5.2 Halo with pavé shank

A popular Indian mid-range design — cushion-cut center, halo, pavé running down the shoulders.

piece: ring
  shank:
    type: tapered
    shoulder_width_mm: 2.4
    base_width_mm: 1.8
  head:
    type: halo_single
    halo_stone_count: 14
    halo_stone_size_mm: 1.2
    prong_style: claw_round
  center_stone:
    cut: cushion
    length_mm: 6.5
    width_mm: 6.5
    depth_mm: 4.2
    carat_weight: 1.25
  accent:
    type: pave
    stone_size_mm: 1.2
    row_count: 1
    length_mm: 14.0
    placement: shoulder_both_sides
  gallery:
    type: filigree
  finish:
    type: high_polish
  metal:
    material: 18k_rose_gold
    tone_variation: single_tone
  size:
    standard: IN_13
    resizable: true
    resize_range: [IN_10, IN_16]

## 5.3 South Indian temple-style solitaire

Traditional design with floral gallery, tiger-claw prongs, milgrain edge. Typical of Tamil Nadu and Kerala retail.

piece: ring
  shank:
    type: milgrain_edge
    width_mm: 2.8
    thickness_mm: 2.0
    milgrain_size: medium
  head:
    type: solitaire_4prong
    prong_style: tiger_claw
    prong_length_mm: 4.0
  center_stone:
    cut: round_brilliant
    diameter_mm: 5.5
    carat_weight: 0.70
  gallery:
    type: floral
  finish:
    type: high_polish
  metal:
    material: 22k_yellow_gold
    tone_variation: single_tone
  size:
    standard: IN_14
    resizable: true
    resize_range: [IN_11, IN_17]

# 6. The Vision Parser

Vision model input: one photo of a ring. Output: a JDL document with per-field confidence scores.

## 6.1 Parser prompt structure

The parser is not trained. It is a structured-output prompt to a frontier VLM (Claude Vision, GPT-4V, Gemini). The prompt includes the full JDL vocabulary as the output schema. The model classifies — it does not generate geometry.

INPUT: [photo]
SCHEMA: [full JDL vocabulary tables from Section 3]
INSTRUCTION: Identify each JDL component visible in this ring.
For each field, return:
  - value: one token from the vocabulary
  - confidence: 0.0-1.0
  - evidence: short phrase describing what you see

      
If a field cannot be determined from the photo, return
  value: null
  confidence: 0.0
  reason: why (e.g. "side view required", "stone hidden behind prong")

## 6.2 Confidence routing

- confidence ≥ 0.85 on all fields → synthesize CAD immediately

- confidence ≥ 0.85 on structural fields (shank, head, stone), < 0.85 on details → synthesize with defaults, flag details for jeweler confirmation

- confidence < 0.85 on structural fields → request additional photos (side view, top view) via WhatsApp

- confidence < 0.5 on any structural field → route to human CAD designer (fallback tier)

# 7. Open Questions for the Master Jeweler

This is where v0.1 needs correction. Specific questions to bring to a Jaipur or Mumbai master CAD designer:

- Shank coverage: Are there common shank styles in Indian retail missing from the 15 above? Particularly regional — Bengali antique, Punjabi kundan-base, Nizami?

- Head styles: Is the halo/solitaire/bezel taxonomy sufficient, or are there meaningful sub-categories (e.g., hidden halo, compass set) that deserve their own primitives?

- Prong counts: Is counting prongs as a parameter of head (instead of separate primitives per count) the right abstraction?

- Stone seat geometry: Does stone seat design vary enough across head types that it needs its own primitive, or can it be derived from stone cut + head type?

- Multi-stone pieces: How do 3-stone, 5-stone, and trilogy designs decompose? Extend center_stone to be a list, or add a new component type?

- Cultural/religious elements: Om, Ganesha, cross, kalash, peacock — are these rings (in which case they extend the head vocabulary) or pendants (separate piece type)?

- Kundan, polki, jadau: These are traditional Indian setting techniques that don't fit the Western prong/bezel/pavé taxonomy. Do they need their own setting_technique dimension?

- Manufacturing envelopes: Are the wall thickness / feature size numbers in Section 4.2 correct for lost-wax casting at typical Indian foundries? What do they need to be for CNC vs casting vs 3D-printed masters?

- Missing dimensions: What parameters do jewelers routinely modify that v0.1 has not captured? (e.g., shoulder slope angle, gallery height, head-to-shank transition radius)

# 8. From Document to Product

## 8.1 Month 1 — Validate the grammar

- Hire master CAD designer in Jaipur/Mumbai on contract

- Walk through this v0.1 spec, mark every gap and error

- Collect 200 real engagement ring photos (from existing JewelAI jeweler clients). Parse each one manually into JDL. Find everything the grammar cannot express.

- Ship JDL v0.2 by end of month

## 8.2 Month 2 — Build the first 10 primitives

- In CadQuery (Python) or Grasshopper (Rhino)

- Start with: plain_round shank, solitaire_4prong head, claw_round prong, round_brilliant stone seat, high_polish finish, 18k_yellow_gold metal

- Each primitive: parametric, watertight, manufacturability-validated, volume function

- Goal: synthesize example 5.1 end-to-end, send to casting house, receive physical ring

## 8.3 Month 3 — The parser and the pipeline

- Wire Claude Vision / GPT-4V to the JDL schema

- Test parsing accuracy on the 200 photos from Month 1 (now with known-good JDL ground truth)

- Build the confidence routing logic from Section 6.2

- WhatsApp integration: photo in, preview + quote out, payment, CAD out

## 8.4 Months 4–6 — Expand and deploy

- Add the remaining primitives (30–50 total for engagement rings)

- Partner with one casting house for end-to-end fulfillment

- Onboard first 10 paying jewelers (existing JewelAI users)

- Extend JDL to pendants, earrings, bangles

## 8.5 Month 12 and beyond — Make JDL the standard

- Publish JDL spec publicly (the spec, not the primitive library)

- License the synthesizer to Tanishq, Kalyan, Malabar as white-label infrastructure

- Offer JDL export from JewelAI and JDL import to casting houses, appraisers, insurers

- The moat: you own the reference implementation, the most complete primitive library, and the data flywheel of what jewelers actually build

# 9. One Sentence

Stop trying to generate jewelry. Learn to read it, then recompose it.